DELIVERABLE REQUIREMENT: TM Datamining and Loading

CLIENT: RTK

SITE: RTK

PROJECT: TM Full Implementation

REVISION: 1.0

Contents

1.	Purpose	2

2.	TM Datamining & Loading	2

A.	Key Aspects & Features	2

B.	Out of Scope	2

C.	Final Format	2

D.	Assumptions	2

E.	Thickness Monitoring (TM)	3

# Purpose

Pinnacle will use the process and methodology documented in the deliverable requirement TM Datamining & Loading to:

  - Datamine the necessary fields required for the Thickness Monitoring (TM) Module to manage inspections, monitor asset health, and ensure inspection compliance in APM.

  - Develop load sheets for fixed equipment and piping circuits which will provide a baseline reference at the time of the implementation and may be used to identify changes made during Evergreening of the MI Inspection Program

  - Import load sheets into the APM TM Module

# TM Datamining & Loading

## Key Aspects & Features

- Mechanical Data Mining (Equipment and Piping): Pinnacle will collect asset design and construction information from gathered asset documentation. Pinnacle will datamine piping mechanical data using RTK piping standards/specifications.

- Fixed Equipment and Piping Circuit APM Load: Pinnacle will develop load sheets for equipment and piping circuits to load into APM.

  - GE APM Thickness Monitoring (TM) Module

    - Pinnacle will datamine existing historical TML data for equipment and piping circuits, and load to corresponding TMLs in APM.

## Out of Scope

- APM fields not listed as required fields in Section E and Section F and their dependents, will not be datamined and loaded.

- Equipment and Units not listed as in-scope on the MALs will not have their TMLs datamined.

- TMLS that cannot be correlated using an equipment drawing will not be datamined.

## Final Format

Datamined load sheets will be loaded into the GE APM Thickness Monitoring (TM) Module

## Assumptions

- Pinnacle assumes that accurate line specification sheets are available for piping-datamining.

- Pinnacle will only need to datamine the appropriate data as detailed in Section E and Section F of this deliverable requirements document.

- Pinnacle assumes that all asset documentation is accurate and available to the project team.

- Pinnacle is not calculating or assessing for fitness for services (FFS).

- Pinnacle is not performing T-min calculations internal or external to the APM system as part of this scope of work for pressure or structural T-min analysis.

- Minimum Structural T-min for piping will follow API 574 Table 7 for all pipe sizes. T-min for pipe sizes not included in API 574 Table 7 will be modeled after the largest pipe size in Table 7. This will be used for all piping materials at all temperatures.

- All assets are built to ASME VIII, ASME B31.3 or API 650 codes.

- Any non-required field will not be modified.

- Pinnacle will provide a list of TML/Circuit data gaps for RTK to vet gaps and fill in necessary data for Pinnacle to load if desired.

- APM piping formula A will be loaded for the purpose of T-min calculations for piping because it is the most consistent.

## Thickness Monitoring (TM)

Thickness Monitoring Location Load Sheet Fields

Data load template worksheet: Assets

| Load Sheet Field | Populate | Data Requirements |
| --- | --- | --- |
| Equipment ID | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM/SAP Field Details: Text Entry Example Inputs: - 000000000002000092 |
| CMMS System | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM/SAP Field Details: Text Entry Required Input: ECP-500 |
| Equipment Technical Number | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM Field Details: Text Entry Example Inputs: - 3330-TK-003 |

Data load template worksheet: Asset_CAS

| Load Sheet Field | Populate | Data Requirements |
| --- | --- | --- |
| Equipment ID | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM Field Details: Text Entry Example Inputs: - 000000000002000092 |
| CMMS System | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM Field Details: Text Entry Required Input: ECP-500 |
| Equipment Technical Number | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM Field Details: Text Entry Example Input: 3330-TK-003 |
| Inspection Type | Yes | Expected Source: APM Field Details: Text Entry Required Input: UT or RT |
| Controlling Corrosion Rate | Yes | Required Input: Maximum |
| Comments | No | Required Input: Leave Blank |
| Apply CCR to TML Rem Life Next Insp Date Calc | No | Required Input: Leave Blank |
| Safety Factor | No | Required Input: Leave Blank |
| Minimum Number of TMLs | No | Required Input: Leave Blank |
| Percentile | No | Required Input: Leave Blank |
| Std Deviation Factor | Yes | Purpose: Displays the value that determines where the upper and lower limit will be drawn on the asset analysis plot; "2" means that the upper and lower limits will be drawn to represent 2 standard deviations Field Details: Numeric Required Input: 2 |
| Remaining Life Critical Number of TMLs | Yes | Purpose: Displays the number of TMLs to use when calculating Remaining life; 1 means the system will select the most conservative (shortest) remaining life value; selecting more than 1 will result in an average calculation Field Details: Numeric Required Input: 1 |
| Corrosion Rate Options - Least Squares | Yes | Required Input: False |
| Corrosion Rate Options - Short Term | Yes | Required Input: True |
| Corrosion Rate Options - Long Term | Yes | Required Input: True |
| Corrosion Rate Options - Custom A | Yes | Required Input: False |
| Corrosion Rate Options - Custom B | Yes | Required Input: False |
| Default Inspection Interval (Months) | Yes | Purpose: The value that will be used for calculating Interval next inspection date for a TML Field Details: Numeric Example Inputs: 24, 36, 48, 60, 120 |
| Inspection Interval Options - Factor Remaining Life | Yes | Purpose: Determines which inspection dates should be calculated for TMLs and how scheduled next inspection date is determined Field Details: Text Entry Equipment: True |
| Inspection Interval Options - Inspection Interval | Yes | Purpose: Determines which inspection dates should be calculated for TMLs and how scheduled next inspection date is determined Field Details: Text Entry Equipment: True |
| Default T-Min (Inches) | Yes | Purpose: Value used for T-min if field not populated Field Details: Numeric |
| Minimum Corrosion Rate (Mils/year) | Yes | Expected Source: Use DMR corrosion model Field Details: Text Example Inputs: 5 (Mils/year) - Notes: If there is more than 1 modeled TML group for the Asset, utilize the largest internal corrosion rate for the Asset. |
| Use Minimum Corrosion Rate | Yes | Purpose: If checked, the system will use the minimum corrosion rate if the calculated corrosion rate is less than the minimum. Field Details: Text Entry Required Inputs: True |
| Remaining Life Factor | Yes | Purpose: Calculates the remaining life for the equipment by multiplying the rate by the date (0.5 = half life) Field Details: Text Required Inputs: 0.5 |
| Allowable Measurement Variance (inches) | Yes | Purpose: How much difference is allowed between the measurements entered and the two most recent measurements Field Details: Numeric Required Inputs: 0 |
| Corrosion Rate Variance | Yes | Purpose: The amount of variation that is allowed between the short term rate for a TML and the Minimum corrosion rate. If the value is zero, the validation will not be performed. Field Details: Numeric Required Inputs: 0 |

Data load template worksheet: TML_Group

| Load Sheet Field | Populate | Data Requirements |
| --- | --- | --- |
| Equipment ID | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM Field Details: Text Entry Example Inputs: - 000000000002000083 |
| CMMS System | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM Field Details: Text Entry Required Input: ECP-500 |
| Equipment Technical Number | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM Field Details: Text Entry Example Input: 3330-TK-009 |
| TML Group ID | Yes | Purpose: Loaded value for new TML Group Expected Source: CML Datamining Spreadsheet Field Details: Text Entry Example Inputs: - Piping: PIPE - Equipment: 3330-TK-009, 3330-TK-009 - Notes: ≤ 50 characters in length |
| Description | No | Required Input: Leave Blank |
| Corrosion Group | Yes | Purpose: Loaded value for new TML Group Expected Source: CML Datamining Spreadsheet Field Details: Text Entry Example Inputs: - Equipment: SHELL, NOZZLES, ROOF, FLOOR, etc - Piping: PIPE |
| Comments | No | Required Input: Leave Blank |

Data load template worksheet: TML_Group_CAS

| Load Sheet Field | Populate | Data Requirements |
| --- | --- | --- |
| Equipment ID | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM Field Details: Text Entry Example Inputs: - 000000000002000095 |
| CMMS System | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM Field Details: Text Entry Required Input: ECP-500 |
| Equipment Technical Number | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM Field Details: Text Entry Example Input: 3330-TK-008 |
| Inspection Type | Yes | Expected Source: APM Field Details: Text Entry Required Input: UT or RT |
| TML Group ID | Yes | Purpose: Loaded value for new TML Group Expected Source: CML Datamining Spreadsheet Field Details: Text Entry Example Inputs: - Piping: PIPE - Equipment: 3330-TK-009, 3330-TK-009 |
| Controlling Corrosion Rate | Yes | Required Input: Maximum |
| Comments | No | Required Input: Leave Blank |
| Apply CCR to TML Rem Life Next Insp Date Calc | No | Required Input: Leave Blank |
| Safety Factor | No | Required Input: Leave Blank |
| Minimum Number of TMLs | No | Required Input: Leave Blank |
| Percentile | No | Required Input: Leave Blank |
| Std Deviation Factor | Yes | Purpose: Displays the value that determines where the upper and lower limit will be drawn on the asset analysis plot; "2" means that the upper and lower limits will be drawn to represent 2 standard deviations Field Details: Numeric Required Input: 2 |
| Remaining Life Critical Number of TMLs | Yes | Purpose: Displays the number of TMLs to use when calculating Remaining life; 1 means the system will select the most conservative (shortest) remaining life value; selecting more than 1 will result in an average calculation Field Details: Numeric Required Input: 1 |
| Corrosion Rate Options - Least Squares | Yes | Required Input: False |
| Corrosion Rate Options - Short Term | Yes | Required Input: True |
| Corrosion Rate Options - Long Term | Yes | Required Input: True |
| Corrosion Rate Options - Custom A | Yes | Required Input: False |
| Corrosion Rate Options - Custom B | Yes | Required Input: False |
| Default Inspection Interval (Months) | Yes | Purpose: The value that will be used for calculating Interval next inspection date for a TML Field Details: Numeric Example Inputs: 24, 36, 48, 60, 120 |
| Inspection Interval Options - Factor Remaining Life | Yes | Purpose: Determines which inspection dates should be calculated for TMLs and how scheduled next inspection date is determined Field Details: Text Entry Equipment: True |
| Inspection Interval Options - Inspection Interval | Yes | Purpose: Determines which inspection dates should be calculated for TMLs and how scheduled next inspection date is determined Field Details: Text Entry Equipment: True |
| Default T-Min (Inches) | Yes | Purpose: Value used for T-min if field not populated Field Details: Numeric |
| Minimum Corrosion Rate (Mils/year) | Yes | Expected Source: Use DMR corrosion model Field Details: Text Example Inputs: 5 (Mils/year) |
| Use Minimum Corrosion Rate | Yes | Purpose: If checked, the system will use the minimum corrosion rate if the calculated corrosion rate is less than the minimum. Field Details: Text Entry Required Inputs: True |
| Remaining Life Factor | Yes | Purpose: Calculates the remaining life for the equipment by multiplying the rate by the date (.5 = half life) Field Details: Text Required Inputs: 0.5 |
| Allowable Measurement Variance (inches) | Yes | Purpose: How much difference is allowed between the measurements entered and the two most recent measurements Field Details: Numeric Required Inputs: 0 |
| Corrosion Rate Variance | Yes | Purpose: The amount of variation that is allowed between the short term rate for a TML and the Minimum corrosion rate. If the value is zero, the validation will not be performed. Field Details: Numeric Required Inputs: 0 |

Data load template worksheet: TML

| Load Sheet Field | Populate | Data Requirements |
| --- | --- | --- |
| Equipment ID | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM Field Details: Text Entry Example Inputs: - 000000000002000095 |
| CMMS System | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM Field Details: Text Entry Example Input: ECP-500 |
| Equipment Technical Number | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM Field Details: Text Entry Example Input: 3330-TK-012 |
| TML Group ID | Yes | - Purpose: Associated TML Group Record Identification - Expected Source: Equipment Data Collection Spreadsheet - Field Details: Text Entry - Example Input: - Piping: PIPE - Equipment: 3330-TK-009, 3330-TK-009 |
| TML_ID | Yes | Purpose: Loaded value for New TMLs Expected Source: Equipment Data Collection Spreadsheet Field Details: Text Entry Example Input: UT1.001, RT2.004, UTS.002, RT1.009 |
| TML Analysis Type | Yes | Expected Source: APM Field Details: Text Entry Required Input: UT or RT |
| Status Indicator | Yes | Purpose: Statues of measurement in APM for corrosion rate calculations Expected Source: CML Datamining Template Field Details: Text Inputs: Active or Inactive |
| Component Type | Yes | Purpose: Loaded value for New TMLs. Expected Source: CML Data Collection Spreadsheets Field Details: Text ; Character Limit: 10 Example Input: SHELL, REDUCER, NOZZLE, ELBOW |
| Location | Yes | Expected Source: Directional description Field Details: Text Entry Example Input: North |
| Line Number Designation | No | Required Input: Leave Blank |
| ISO Drawing Number | Yes | Expected Source: Equipment Number/Pipe Circuit Number-ISO # Field Details: Text Entry Example Inputs: - 3330-TK-009_SHT01, 330-M-080 |
| Access | Yes | Expected Source: ISO Drawing/Field Sketch Field Details: Text Entry Example Inputs: SCAFFOLD, LADDER, etc Note: These values must adhere to the APM picklist. |
| Inspection Sequence Number | No | Required Input: Leave Blank |
| Insulated Flag | Yes | Expected Source: CML Data Collection Spreadsheets Field Details: Text Entry Example Input: TRUE or FALSE Note: Must be in all caps. |
| TML Comment | No | Required Input: Leave Blank |
| Minimum Thickness | Yes | Expected Source: CML Data Collection Spreadsheets Field Details: Text Entry Example Input: 0.1 - Equipment: - 1. Below 0.1, use 0.1 - 2. Above 0.1, use calc Tmin on PV calc sheet. - 3. Tmin not provided, document in Data Gap Log. - 4. For equipment with nominal below 0.1, utilize nominal for the Tmin. - Piping: - 1. API 574 Table Structural Alert (left side of table) is lowest value TML can be - 2. Tmin not provided, document in Data Gap Log. - Table 7 in API 574 will be applied to all piping materials at all temperatures. - Tanks: - Below 0.1, use 0.1 - Above 0.1, use calc Tmin [Internal Inspection docs have values for Min thickness per Shell Course and a lot of cases even the roof] - Tmin not provided, document in Data Gap Log. - For equipment & ASTs with nominal below 0.1, utilize nominal for the Tmin. |
| Additional Thickness | No | Required Input: Leave Blank |
| Excluded from Analysis | No | Required Input: Leave Blank |
| Number of Readings | No | Required Input: Leave Blank |
| Design Code | Yes | Field Details: Picklist - Piping – B31.3 - Vessel – ASME VIII DIV 1 - Tank – API 650 - *UL142 API 653 |
| Code Year (T-Min Formula) | No | Required Input: Leave Blank |
| Code Year (Allowable Stress Lookup) | No | - Required Input: Leave Blank |
| Material Specification | No | - Required Input: Leave Blank |
| Material Grade | No | - Required Input: Leave Blank |
| Material Type | No | - Required Input: Leave Blank |
| Design Pressure | No | - Required Input: Leave Blank |
| Design Temperature | No | - Required Input: Leave Blank |
| Nominal Thickness | Yes | Expected Source: CML Data Collection Spreadsheets Field Details: Numeric Example Input: 0.154 |
| Override Allowable Stress | Yes | Required Input: False |
| Allowable Stress | No | Required Input: Leave Blank |
| Outside Diameter | No | Required Input: Leave Blank |
| Inside Diameter | No | Required Input: Leave Blank |
| Joint Factor | No | - Required Input: Leave Blank |
| Corrosion Allowance | Yes | Purpose: Loaded value for New TMLs. Expected Source: Data Collection Spreadsheets Field Details: Numeric |
| Piping Nominal Diameter - NPS | Yes, Piping | Purpose: Loaded value for New TMLs. Expected Source: Data Collection Spreadsheets Field Details: Load sheet Picklist Example Input: 6 |
| Piping Nominal Diameter - DN | Yes, Piping | Purpose: Looks up the wall thickness & will find the structural thickness to support cascading Picklist which ties to NPS & Schedule Expected Source: Data Collection Spreadsheets Field Details: Load sheet Picklist Example Input: 6 |
| Schedule | No | Required Input: Leave Blank |
| Piping Formula | No | Required Input: Leave Blank |
| Mechanical Allowance | No | Required Input: Leave Blank |
| Design Factor | No | Required Input: Leave Blank |
| Temperature Factor | No | Required Input: Leave Blank |
| Tank Type | No | Required Input: Leave Blank |
| Shell Type | No | Required Input: Leave Blank |
| API Method | No | Required Input: Leave Blank |
| Plate Thickness | No | Required Input: Leave Blank |
| Minimum Yield Strength | No | Required Input: Leave Blank |
| Minimum Tensile Strength | No | Required Input: Leave Blank |
| Course Height | No | Required Input: Leave Blank |
| Course Number | No | Required Input: Leave Blank |
| Fill Height | No | Required Input: Leave Blank |
| Specific Gravity | No | Required Input: Leave Blank |
| Floor Plate has Detection? | No | Required Input: Leave Blank |
| Floor Plate has Reinforced Lining? | No | Required Input: Leave Blank |
| Maximum Operating Fill Height | No | Required Input: Leave Blank |
| Distance from the Bottom | No | Required Input: Leave Blank |
| Tank Diameter | No | Required Input: Leave Blank |
| Vessel Type | No | Required Input: Leave Blank |
| PV Formula | No | Required Input: Leave Blank |
| Dish Radius | No | Required Input: Leave Blank |
| Knuckle Radius | No | Required Input: Leave Blank |
| Apex Angle | No | Required Input: Leave Blank |
| Inside Radius | No | Required Input: Leave Blank |
| Outside Radius | No | Required Input: Leave Blank |
| Head Diameter | No | Required Input: Leave Blank |
| Estimated T-Min? | No | Required Input: Leave Blank |
| Design Stress Constant | No | Required Input: Leave Blank |

Data load template worksheet: TML_CAS

- Do Not Populate

Data load template worksheet: Measurements

| Load Sheet Field | Populate | Data Requirements |
| --- | --- | --- |
| Equipment ID | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM Field Details: Text Entry Example Inputs: - 000000000002000092 |
| CMMS System | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM Field Details: Text Entry Required Input: ECP-500 |
| Equipment Technical Number | Yes | Purpose: Associated Equipment Record Identification Expected Source: APM Field Details: Text Entry Example Input: 3330-TK-003 |
| TML Group ID | Yes | - Purpose: Associated TML Group Record Identification - Expected Source: Equipment Data Collection Spreadsheet - Field Details: Text Entry - Example Input: - Piping: PIPE - Equipment: 3330-TK-009, 3330-TK-009 |
| TML_ID | Yes | Purpose: Loaded value for New TMLs Expected Source: Equipment Data Collection Spreadsheet Field Details: Text Entry Example Input: UT1.001, RT2.004, UTS.002, RT1.009 |
| Readings | Yes | Purpose: Measurement Reading Expected Source: UT Reports Field Details: Numeric Example Input: 0.256 |
| Measurement Date | Yes | Purpose: Measurement Taken Date Expected Source: UT Reports Field Details: Short Date Example Input: 3/15/2020 |
| Nominal | Yes | Purpose: Identifies Nominal Reading Expected Source: Remaining Life Spreadsheets Field Details: Text Inputs: True or False |
| Measurement Status Indicator | Yes | Purpose: Statues of measurement in APM for corrosion rate calculations Expected Source: CML Datamining Template Field Details: Text Inputs: Active or Inactive |
| Measurement Taken By | No | Required Input: Leave Blank |
| Measurement Comments | No | Required Input: Leave Blank |
| Temperature | No | Required Input: Leave Blank |