Quantitative Reliability Optimization (QRO)

Complete Operational Guide, Workflows & Methodological Architecture

1. Understanding the Industrial Context

Industrial facilities—refineries, chemical plants, lumber mills, pharmaceutical plants, mining operations, wastewater treatment facilities, and many others—all share one fundamental characteristic:

they rely on thousands of interconnected, complex, and expensive assets that must function together for the facility to operate safely and profitably.

When conditions are normal and markets are stable, most facilities operate safely and generate significant margins. But the failure of any individual asset in this network—whether a pump, a pipeline segment, a heat exchanger, a conveyor, or a reactor—can have immediate consequences:

Lost production

Safety incidents or injuries

Environmental releases

Community or regulatory impacts

Very high financial losses, sometimes hundreds of thousands or millions of dollars within a few days

Industrial operators therefore aim to optimize their investments in:

Maintenance activities

Reliability programs

Inspection efforts

Spare parts and inventory

Capital improvements

However, making these decisions is extremely challenging when thousands of assets and countless uncertainties are involved.

2. The Historical Approaches and Their Limitations

For decades, facilities have relied on structured programs to manage this complexity—most notably:

Reliability-Centered Maintenance (RCM):

Originated in the 1970s

Formalized with major publications in the 1980s

Designed to systematically determine what maintenance tasks should exist for a given asset

Highly dependent on human judgment and team (consensus) decision making

Slow, subjective, and often conservative

Risk-Based Inspection (RBI):

Developed in the 1990s by industry experts

Widely used in refining and petrochemical industries

Provides a risk-informed approach for inspection planning

Still heavily reliant on expert opinion

Typically siloed within “fixed equipment” and not integrated across all asset types

While RCM and RBI were important advancements for their time, they share several limitations:

Heavy reliance on subjective human decision-making

Slow and labor-intensive

Often overly conservative, leading to unnecessary tasks and spending

Siloed execution, with fixed equipment, electrical, rotating machinery, and maintenance teams all working independently

Limited use of modern data, ignoring readily available operational and condition-based datasets

Poor system-level prioritization, causing facilities to overinvest in some areas while missing critical risks in others

As a result, even the best-run facilities still experience avoidable failures and significant wasted effort, amounting to hundreds of millions of dollars annually.

3. The Origin and Development of QRO

QRO was created to overcome the limitations of traditional approaches. Over the past decade, Pinnacle developed QRO through a combination of:

Advanced machine learning algorithms

First-principles engineering

Extensive real-world industrial data

The core idea behind QRO is straightforward:

If a facility systematically combines operational data, inspection data, physics-based degradation models, and statistical techniques, it should be able to predict asset performance far more accurately than human-driven methods allow.

QRO was developed through years of:

Designing algorithms

Testing them against real plant data

Iterating models to incorporate first-principles physics (e.g., corrosion mechanisms, temperature effects, material properties)

Validating predictions against actual asset histories

Proving that quantitative, data-driven models outperform human subjective estimates

Workflow 1: Base Model Workflow

Make a tenant

⇣

Asset_Register Build one in the front end all the dummy to placeholders from area to Assessment; ➔; Export Asset Register from Newton; ➔; Populate Asset Register; ➔; Import Asset Register; ➔; Troubleshoot Asset Register Errors.

↓

Build_System_Model Build Linear RBD Unit System Model; ➔; Run 'Validate' RBD Unit System Model; ➔; Troubleshoot RBD Unit System Model Errors; ➔; Set RBD Unit System Model to Active. Build Linear RBD Facility System Model; ➔; Run 'Validate' RBD Facility System Model; ➔; Troubleshoot RBD Facility System Model Errors; ➔; Set RBD Facility System Model to Active.

↓

Inspection_Data Correlate CMLs/Monitoring to Assessment; ➔; Export Inspection Data from Newton; ➔; Populate Inspection Data; ➔; Import Inspection Data; ➔; Troubleshoot Inspection Data Errors.

⇣

PoF Configure Damage Modes and Damage Mechanisms to match client's; ➔; Build one POF in the front end; ➔; Export PoF Assessment Data from Newton; ➔; Populate PoF Assessment Data; ➔; Import PoF Assessment Data; ➔; Troubleshoot PoF Assessment Data Errors.

⇣

Task Export Asset Register; ➔; Export Asset Task Data; ➔; Copy Assessment from Asset Register to Asset Task Data; ➔; Populate Replace task to Asset Task Data; ➔; Import Asset Task Data; ➔; Troubleshoot Asset Task Data Errors.

↓

Calculate Export PoF Assessment Data; ➔; Change Assessment status to 'ACTIVE'; ➔; Import PoF Assessment Data.

↓

Troubleshoot and QC

↓

Base Model Complete

Workflow 2: Tuned Model Development Workflow

| ◆ Base | ➔ | Define Customer Performance Objectives & Goals | ➔ | Load Current Planned Tasks into Newton™ | ➔ | Load COF Data into Newton™ | ➔ | Assess Top 10 Contributors to Downtime Validate Newton™ Model with Customer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

↓ (Branches to Tuning Tracks & Scenario Creation)

Understand Tuning Needed to Align Newton™ Model with Reality; ➔; Tuning Toolbox; ➔; Close Tuning Needs.

Create New Scenario for Newton™ Tuned Model; ➔; ◆ Tuned.

Understand Foundational Program Gaps to Close to Align Model; ➔; Uncertainty Based Gap Closure; ➔; Close Foundational Program Gaps.

| Workflow 3: CML Optimization Setup Load Asset Register At least one child component. ↓ Assign Failure Mode Thinning — Multi-Path Linear. ↓ Configure Failure Mechanism Sets % Affected Area on all CMLs. ↓ Load CoF Data At least HSE or Economic CoF. ↓ ◆ Using customer's existing inspection tasks?. ↙ [YES: Load Planned Tasks] ¦ ↘ [NO: Run Task Automation] Load Planned Inspection Tasks (Dates set inspection); Run Task Automation (Newton generates tasks). ↓ (Also requires: Load CMLs & Readings ➔ Run Newton Calculation) Ready to Run CML Optimization. | Workflow 4: Running CML Optimization Execution Navigate Optimize menu ➔ CML Optimization. ↓ Validate Data Click Validate Data button. ↓ ◆ Errors found?. [Yes] Fix errors in assessment data ↺; [No] Proceed to Configure. ↓ Start and Configure Run • Optimization Date • Economic Risk Threshold • HSE Risk Threshold. ↓ Execute Click Run / Execute. ↓ ◆ Server validation passes?. [Errors] Review error messages and fix data ↺; [Passes] Review Results ✔. |
| --- | --- |

5. Model Progression Lifecycle

5.1 Stage 1: Base Model Generation

The process begins by loading into Newton:

The accepted "Master Asset List" for the system being modeled.

A simplified System Model for the assets being included.

Next, the team loads any readily available data (i.e. data that maintenance, reliability or operations personnel would have easy access to in normal conditions):

thickness data

vibration data

failure modes (documented)

Process and operating information

Relevant system model parameters (downtime)

Newton then produces an initial base model, which represents what the data currently says about asset risk and degradation.

These base models are almost always inaccurate at first—not because of QRO, but because:

Data is missing

Data contains errors

Asset records are incomplete

Inputs have never been standardized across systems

Human-driven systems allowed data quality issues to propagate for years

This initial model is invaluable because it immediately shows where the data is wrong or insufficient, and what should be done to get initial priorities from the model.

5.2 Stage 2: Tuned Model Development

Next, the engineering and operations teams collaborate to refine the model by:

Correcting data issues

Incorporating missing data streams

Adjusting inputs to reflect real operating behavior

Applying additional physics-based rules

Cleaning up or reconciling historical records

This stage results in a tuned model—a digital representation of the facility that accurately reflects real-world performance.

Once the tuned model resembles reality, QRO can then be used to evaluate improvements.

5.3 Stage 3: Optimized Model Generation

The final stage is to create the optimized model, where QRO determines:

Which tasks or activities should be performed

How frequently they should occur

How they should be prioritized against each other

What the return on investment is for each task

How to achieve the greatest total risk reduction at the lowest total cost

This optimized model becomes a practical, executable plan that operators can deploy directly into the facility. It enables:

Maximum reliability

Minimum cost

Highest safety performance

Better alignment of maintenance, inspection, and engineering activities

Data-driven capital planning

This is the stage where QRO fully demonstrates its value—providing a strategic, financially optimized roadmap for improving facility operations.